University of jordan

School of engineering

Mechanical department

Strength Lab Report

Experiment #8

Strain gauge

Introduction

A strain gage (sometimes referred to as a Strain gauge) is a sensor whose resistance varies with applied force; It converts force, pressure, tension, weight, etc., into a change in electrical resistance which can then be measured. When external forces are applied to a stationary object, stress and strain are the result. Stress is defined as the object's internal resisting forces, and strain is defined as the displacement and deformation that occur. 

The strain gage is one of the most important sensor of the electrical measurement technique applied to the measurement of mechanical quantities. As their name indicates, they are used for the measurement of strain. As a technical term "strain" consists of tensile and compressive strain, distinguished by a positive or negative sign. Thus, strain gages can be used to pick up expansion as well as contraction.

Objective :

To compare the experimental results of stress and strain resulting from the strain gauge experiment with theoretical results.

Apparatus

Theory : The strain gage is one of the most widely used strain measurement sensors. It is a resistive elastic unit whose change in resistance is a function of applied strain. where R is the resistance,  is the strain, and S is the strain sensitivity factor of the gage material (gage factor in some books). Among strain gages, an electric resistance wire strain gage has the advantages of lower cost and being an established product. Thus it is the most commonly used type of device. Other types of strain gages are acoustic, capacitive, inductive, mechanical, optical, piezo-resistive, and semi-conductive. A wire strain gage is made by a resistor, usually in metal foil form, bonded on an elastic backing. Its principle is based on fact that the resistance of a wire increases with increasing strain and decreases with decreasing strain, as first reported by Lord Kelvin in 1856. Consider a wire strain gage, as illustrated above. The wire is composed of a uniform conductor of electric resistivity  with length l and cross-section area A. Its resistance R is a function of the geometry given by The resistance change rate is a combination effect of changes in length, cross-section area, and resistivity. When the strain gage is attached and bonded well to the surface of an object, the two are considered to deform together. The strain of the strain gage wire along the longitudinal direction is the same as the strain on the surface in the same direction. However, its cross-sectional area will also change due to the Poisson's ratio. Suppose that the wire is cylindrical with initial radius r. The normal strain along the radial direction is The change rate of cross-section area is twice as the radial strain, when the strain is small. The resistance change rate becomes For a given material, the sensitivity of resistance versus strain can be calibrated by the following equation. When the sensitivity factor S is given, (usually provided by strain gage vendors) the average strain at the point of attachment of the strain gage can be obtained by measuring the change in electric resistance of the strain gage.

Collected data :

Table 1

|  |  | Bending | Experiment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Load in N | 0 | 1 | 2 | 3 | 4.5 | 5.5 |
| Reading in Mv/V | 0 | 0.035 | 0.71 | 0.105 | 0.157 | 0.192 |

Table 2

|  |  | Tensile | Experiment |  |  |
| --- | --- | --- | --- | --- | --- |
| Load in N | 0 | 10 | 20 | 30 | 40 |
| Reading in Mv/V | 0 | 0.005 | 0.008 | 0.012 | 0.015 |

Table3

|  |  | Torsion | Experiment |  |  |
| --- | --- | --- | --- | --- | --- |
| Load in N | 0 | 5 | 10 | 15 | 20 |
| Reading in Mv/V | 0 | 0.035 | 0.07 | 0.104 | 0.139 |

Calculated data :

Bending experiment :

Table 4

| Load in N | 0 | 1 | 2 | 3 | 4.5 | 5.5 |
| --- | --- | --- | --- | --- | --- | --- |
| Reading in mV/V | 0 | 0.035 | 0.071 | 0.105 | 0.157 | 0.192 |
| exp strain (mm) | 0 | 0.0170732 | 0.034634 | 0.05122 | 0.076585 | 0.093659 |
| exp stress (Mpa) | 0 | 3.5853659 | 7.273171 | 10.7561 | 16.08293 | 19.66829 |
| theo stress (Mpa) | 0 | 3.3661769 | 6.732354 | 10.09853 | 15.1478 | 18.51397 |
| theo strain (mm) | 0 | 0.0160294 | 0.032059 | 0.048088 | 0.072132 | 0.088162 |
| error % |  | 6.5115091 | 8.033102 | 6.511509 | 6.173377 | 6.234856 |

Torsion experiment :

Table 5

|  |  | Torsion | Experiment |  |  |
| --- | --- | --- | --- | --- | --- |
| Load in N | 0 | 5 | 10 | 15 | 20 |
| Reading in mV/V | 0 | 0.035 | 0.07 | 0.104 | 0.139 |
| exp strain (mm) | 0 | 0.017073 | 0.034146 | 0.050732 | 0.067805 |
| exp stress (Mpa) | 0 | 2.731707 | 5.463415 | 8.117073 | 10.84878 |
| theo stress (Mpa) | 0 | 2.547771 | 5.095541 | 7.643312 | 10.19108 |
| theo strain (mm) | 0 | 0.015924 | 0.031847 | 0.047771 | 0.063694 |
| error % |  | 7.219512 | 7.219512 | 6.198374 | 6.453659 |

Tensile experiment :

Table 6

|  |  | Tensile | Experiment |  |  |
| --- | --- | --- | --- | --- | --- |
| Load in N | 0 | 10 | 20 | 30 | 40 |
| Reading in Mv/V | 0 | 0.005 | 0.008 | 0.012 | 0.015 |
| exp strain (mm) | 0 | 0.0071376 | 0.005981 | 0.008971 | 0.011214 |
| exp stress (Kpa) | 0 | 713.952 | 1142.323 | 1713.485 | 2141.856 |
| theo stress (Kpa) | 0 | 500 | 1000 | 1500 | 2000 |
| theo strain (mm) | 0 | 0.002618 | 0.005236 | 0.007853 | 0.010471 |
| error % |  | 19.611 | 14.23231 | 14.23231 | 7.092795 |

Sample calculations :

Bending experiment :

For load = 10N

ɛ ex=1k*VV=12.05*0.005=0.01707

σ ex=E*ɛ=210000*0.01707*10^3=3.58536MPa

σtheoretical=6*F*Lbh2=6*10*0.250.004752*0.01975=3.366MPa

ɛ theoretical=σE=3.366210000=0.01602

torsion experiment :

for load=5 N

ɛ ex=1k*VV=12.05*0.035=0.01707

τ ex=G*2ɛ=80000*2*0.01707*10^3=2.731200MPa

τtheoretical=16*Tπ*d3=6*5*0.13.14*0.012=2.5477 MPa

ɛ theoretical=τ2G=2.54772*80000=0.015923 mm

tensile experiment :

for load =  10 N  µ=0.305   E=191000

ɛ ex=42k*11+µ*VV=22.05*11+0.350*0.005=0.00361336

σ ex=E*ɛ=191000*0.00361 =3.58536MPa

Types of strain gauges :

1. Unbonded metal strain gauges
2. Bonded metal wire strain gauges
3. Bonded metal foil strain gauges
4. Vacuum deposited thin metal film strain gauges
5. Sputter deposited thin metal strain gauges
6. Bonded semiconductor strain gauges
7. Diffused metal strain gauges.

Conclusion:

For bending experiment :

The experimental results obtained were quite close to the theoritical results with an error range (6-6.5%)

For torsion experiment :

Experimental results were also quite close to the theoretical results with an error range of (6-7.5%)

For tension experiment :

Experimental results were also quite close to the theoretical results with an error range of (7-19.6%)